Divide excavation area into 10m x 10m grid squares with a means to identify the grid on site

START

Identify method, Treat and

Re-test

Yes

Do 
samples have 
hazardous properties or substances?

WM3 Appendix B & C

Take one sample from each grid square at each 1m depth of planned excavation

Can treatment remove or reduce hazards?

Yes

Send samples for Composition / Characterisation testing

No

Are 
hazardous 
properties acceptable at

La Collette?

Check Non-Inert

WAC Table 2

No

Hazardous: assign EWC code (WM3 Appendix A) and carry out leach testing

Yes

Non-hazardous: assign EWC code (WM3 Appendix A) and carry out leach testing

No

Are leach results within La Collette hazardous limits? Check Non-Inert WAC Table 2

No

Stabilise or seek alternative disposal

Are leach results within La Collette inert limits?

Check Inert WAC

Table 2

END

Dispose as hazardous

Yes

No

Are leach results within La Collette SNRC limits?

Check Inert WAC

Table 2

No

Yes

END

Dispose as Inert

END

Dispose as Stable Non-Reactive Contaminated

Yes